# Documentation of Differences Between SWMM Version 4.4A and Version 4.31

Charles I. Moore – Camp Dresser & McKee

March 12, 1999

# INTRODUCTION

The following documents the major differences in the source Fortran code for the following two versions of SWMM:

Version 4.4A -	This is the SWMM Version 4.4A program downloaded from Dr. Wayne Huber’s web site on March 9, 1999.  The files were contained in a PKZIP file named SW44CODE.ZIP.  The most recent changes were made to OGATES.FOR on 8/8/97 at 11:51 AM.  Header file describes this program as “Version 4.40 (Lahey, CDM Beta)”.

Version 4.31 -	This is the Version 4.31 code contained in Dr. Huber’s web site on March 9, 1999.  The source code was contained in the PKZIP file SWMMCODE.ZIP.  Latest changes were made to PRPOLL.FOR on 1/23/97 at 15:11.  Header file describes this program as SWMM 4.31 (Lahey, OSU beta).

Most of the changes involve adding additional features that should not result in changes to the model results.  These changes were made to allow the programs to be backward compatible.  Old data set should run on the new program without modification.

Discussions at the end of this memorandum compare model results for a few limited sample cases and summarizes the reasons for these differences.

## PROGRAM CHANGES

Major changes to the program include in the 4.40 version are summarized below in no particular order.  Some of these changes required major code modifications throughout the program source code (e.g., changes to internal numbering of conduit type).  In addition to these major changes, additional changes were made to 1) improve computational efficiency, 2) improve readability of the code, 3) make minor corrections to logic, spelling, etc, 4) improve checks for input data errors, and 5) add or improve messages when input errors occur.  These changes were identified by using the WinDIFF ® program to compare the two source codes line by line.

1 - 	Modification to internal conduit type numbers in EXTRAN.  This change was made to make it easier to add additional line conduit types in the future.  This also simplifies some of the if statements contained throughout the code.  This change affected many subroutines.

2 -	Added three new closed conduit types to EXTRAN:

Vertical Ellipse

Horizontal Ellipse,

Arch.

3 -	Added new weir options to EXTRAN.  These changes are documented in the EXTRAN.DOC file.  Trapezoid weirs are included.  Added option to include correction for sidewall end contractions.  Also added option to use the Villemonte equation for submergence correction.

4 -	Added pump type 4 to EXTRAN.  Type 4 represents variable speed pumps where flows are linearly interpolated from the head in the pumped junctions.  This curve defines a piecewise linear flow versus stage relationship that can be used used to simulate other flow control devices (e.g., vortex valves, complex weirs, pond outlet control structures, etc.).

5 -	Added option to include more than three points on pump curves for all pump types in EXTRAN.

6 -	Added option to limit maximum total infiltration volume in RUNOFF.  The maximum infiltration volume works for both Horton and Green-Ampt.  The available infiltration volume will recover during nonrain periods if the Horton infiltration method is used.  It will not recover if the Green-Ampt equation is used.

7 - 	Option to skip EXTRAN computations (calls to _ROUTE routines) during steady-state periods.  Controls for this option are included on the B0 line.  This is the SWMM ES code.

8 -	Restructured EXTRAN’s DEPTHX subroutine. This change was required to implement change number 1.  Code was changed to make it easier to understand the program logic.  This will reduce chance of programming errors in the future as additional pipe types are added.  This also improves computational efficiency somewhat but should not change model results.

9 - 	Interface to TRANAID from EXTRAN model.  Program will write volumes and flows to a file for input to a water quality model, TRANAID.  This code was written by Dr. Raymond Walton (West Consultants, Inc., Seattle WA)

10 - 	Increase number of rainfall gages (hyetographs) from 10 in RAIN, RUNOFF, and STATS blocks.  The maximum number is defined by a parameter statement.

11 - 	Various changes to HEAD routine in EXTRAN.  Changes were made to improve computational efficiency and to try to make code easier to follow.  Tried to eliminate duplicate if statements and duplication of code.  The calculations performed by this subroutine are not well documented.

12 -	Allow tidal time history to be read to boundary junctions from a separate input file for EXTRAN.

13 - 	Added bridge routine to EXTRAN.  A new close conduit type was added.  The program has inputs to describe the river cross-section at the bridge opening, the number and width of piers, and the shape of the bridge low chord.  The program uses these data to build tables of normalized top-width, area, and hydraulic radius.  Bridges are then treated as other closed conduits in the EXTRAN calculations.

14 -	Intermediate continuity output in EXTRAN.  Model will write continuity summary at specified intervals during the model simulation.  This option was added to make it easier to identify when model runs go unstable when performing long duration simulations.  Continuity is printed in intermediate output and periods with maximum continuity errors are listed in the summary output.

15 - 	Ability to have EXTRAN write flows to ASCII or binary file for specified conduits at specified time steps.  This change provides the option to write results only when flows are greater than user input minimum.  Binary (unformatted sequential) file written in format that allows results to be analyzed with STATS routine.  This decreases the size of the interface file produced by EXTRAN which normally writes flows at all conduits for all time steps.

16 -	Various changes for orifices.  Added rectangular orifices.  Added options to have orifices open and close in response to head at any junction in the system.  Orifice can open from the to (fixed invert) or from the bottom (variable invert).  This option provides a degree of real-time control to the standard model.  Note that an orifice can be place in series with a pump station to simulate real-time control of a pump station.  This option provides a degree of real-time control to the SWMM-EXTRAN program.

17 - 	Read file names for JIN and JOUT from command line if not provided with the @ line.  This eliminates the need to edit file each time a simulation is made with different interface files.

18 -	Simplified rank-order table from STATS block to ease subsequent analysis.  Previous version broke table in to several columns to save paper when printing.  The table now has a single series of columns to ease reading by a post processor or to make it easier to bring results into spreadsheets.

19 -	Added option in STATS block to process all locations in the interface file with single run.  This is implemented by setting KOCRQ or LOCRQ to –1.

20 - 	Summary results scratch file for EXTRAN was changed from ASCII to unformatted sequential.  This speeds writing and reading and makes transfer file smaller.

21 -	Changed EXTRAN to have program not save maximums before NSTART iterations have been complete to prevent spurious startup values from being reported in junction and conduit summary tables.

22 -	Change to surcharged weir calculations in EXTRAN.  This is the case where water depth exceeds YTOP.  In this case, the weir is replaced by an equivalent rectangular conduit.  In version 4.31 and earlier, the rectangular conduit coefficients are set such that the flow in the time step after surcharge occurs is the same as the flow computed before surcharge.  This causes problems when the surcharge is caused by backwater conditions.  In the new model, the equivalent conduit coefficients are set such the flow trough the orifice under free flow conditions is the same as the weir under free flow conditions.  This seems to work well in most cases.  It is recommended that surcharge weirs be avoided if possible and if they cannot be avoided, the user should substitute the weir with an equivalent conduit manually.

COMPARISON OF MODEL RESULTS

Most of the changes listed above involve the EXTRAN block.  Changes in other blocks are minor and should not produce changes in model results.  EXTRAN simulations were made for several sewer system models using executables from the two source programs and the results were compared.  In general, the model results produced by the two programs are similar.  But, there are differences as summarized below:

Continuity errors produce by the program are similar.

Maximum heads reported in the summary tables generally differ by less than 0.05 feet at most junctions.

In some cases, larger differences are reported.  The 4.40 program reports maximum elevation as much as 0.8 feet higher than the 4.31 program.  Closer review of model results show that the reported peak from the 4.40 program occurs when the sewers go into surcharge.  The times of peak for the 4.40 program are generally earlier than the 4.31 program.  The peaks reported by 4.40 are higher than the peak suggested by plots of flows produced by MTVE using five-minute intermediate output intervals.

Based on these observation, and my general experience with the models, it seems that 4.40 is less stable that the 4.31 program when going into and coming out of surcharge iterations.  The peak flow reported in the summary table is a spurious instantaneous peak that occurs as a result of the instabilities associated with going into and coming out of surcharge.  This reported peak elevation overestimates the true maximum water surface elevation.  It is likely that these instabilities may have been introduced by changes made to the EXTRAN HEAD routine.  These problems have been resolved in subsequent versions of the HEAD program.

The identified major model differences are limited to closed conduit systems that go into surcharge.