# Documentation of Differences Between SWMM Version 4.4G and Version 4.4A

Charles I. Moore – Camp Dresser & McKee

March 12, 1999

# INTRODUCTION

The following documents the major differences in the source Fortran code for the following two versions of SWMM:

Version 4.4G  -	This is the SWMM Version 4.4G program prepared by CDM.  This list includes changes made through May 1999.

Version 4.4A -	This is the SWMM Version 4.4A program downloaded from Dr. Wayne Huber’s web site on March 9, 1999.  The files were contained in a PKZIP file named SW44CODE.ZIP.  The most recent changes were made to OGATES.FOR on 8/8/97 at 11:51 AM.  Header file describes this program as “Version 4.40 (Lahey, CDM Beta)”.

The following section summarized the major changes made to the program that are included in the 4.4G program.  The program WinDIFF ® was used to compare the two program source codes line by line to identify and verify the major program changes.  In many cases, the changes required that numerous source code lines be changed throughout the program code.  In addition to these major changes, additional changes were made to improve computational efficiency, readability of the code, and improve program output.

## PROGRAM CHANGES

The following documents major differences between the two source codes.

1 -	Change to print summary output for a number of distinct specified periods during the simulation in EXTRAN.  This option was created to reduce size of output file for long duration simulations.  This is also an aid to debugging a simulation that goes unstable part way into a long duration simulation.

2 -	Ability to apply monthly base flow factors in EXTRAN and TRANSPORT.  Factors are used to adjust constant base flows on D1 lines in EXTRAN.  In TRANSPORT factors are used to adjust constant base flows entered for manhole types (NTYPE 19 or 22) and computed sewage flow.

3 - 	Provide the option to print out all 10 digits or 10 characters of conduit and junction ID’s in EXTRAN.  Requesting this option may cause some graphical post processing programs (e.g., MTVE) to be unable to process EXTRAN output file correctly.  Input data summary tables, intermediate output tables, and summary output tables are reformatted to accept this change.  Also modified RUNOFF and TRANSPORT to print all 10 digits.

4 -	As part of change 3, the number of significant digits for peak and average flows were increased if expanded tables are requested in the output.

5 -	Provide the option to remove intermediate headers from conduit and junction summary tables in EXTRAN.  Also eliminated intermediate headers from tables that echo input data for junctions and conduits.  This eases post-processing of program results such as importing tables into a spreadsheet or database for further analysis or reporting.

6 - 	Switched from Lahey F77L3 compiler to Digital Visual Fortran (DVF) Version 6.0.  Required changes in some routines mainly dealing with 1) reading of file names from command line, 2) functions that get and process system date and time, and 3) string and file handling functions.  Change to get character control to work on writes to screen. Open unit 6 as CON with FORTRAN carriage control in subroutine MAIN.  If carriage control is needed, must write to 6 not “*”.  Compile program with default real type as REAL8 (double precision) and common element alignment as 8.  One drawback of switching is that the interface files are not compatible between DVF and Lahey.  I think that they could be made compatible by using appropriate compiler switches but do not have the time to pursue this.  A complicating factor is the fact that we are using double precision as the default.

7 -	As part of switch to DVF, changed way that input lines that have optional parameters are read.  In new version, STRIP was modified to pad lines with optional inputs with zeros.  This eliminates the need to backspace when errors are found in input routines for all blocks.  This latter process did not seem to always work in DVF.

8 - 	Changes to make SWMM Y2K compatible.

-	If year is entered as two digit (e.g., less than 100) program assumes it is 1900.  This allows program to be backward compatible.

- 	All fields where year was written were expanded were expanded to four digits.

- Program tracks date as a Julian date.  In previous version it was YYDDD where YY was two digit year and DDD was Julian day (069 is March 10 in non-leap years).  Has been increased to YYYYDDD.  Appropriate changes have been made to format of write statements.

- In COMBINE, and Julian date of 99999 or greater signified end of data.  This was changed to 9999999.

- Eliminated locations in RAIN block where 1900 is subtracted from year if year is greater than 1900.

Limited testing suggests that RUNOFF and EXTRAN are Y2K compatible though problems could occur.  Problems should be easily fixed as they arise.  To protect CDM from liability, CDM will not certify that the EPA-SWMM programs are Y2K compatible.

9 -	Implemented a change to correct problem that occurs for M2 and S2 water surface profile conditions in EXTRAN.  This problem can be observed if you set up a simple model with a series of rectangular conduits in series with similar slope, constant inflow to uppermost junction, and free outfall at downstream junction.  If this model is run to a steady-state condition, you will find that depth of flow at upstream end of bottom conduit is greater than depths in upstream conduits.  This “damming effect” results from the way that areas and hydraulic radii at upstream and downstream ends of conduits are averaged to compute flow in a conduit in subroutine HEAD.  In most cases, the resulting overestimation of elevation and depth of flow is small and does not propagate far upstream.  It is important in some cases, such as when modeling combined sewer regulators.  Based on work by Brett Cunningham, a correction was made to the way that areas are averaged.  This seems to correct the model results for simple cases but causes instabilities in complex systems.  Analytical solutions were used to compute corrected weighting factor to use when computing YMID.  This should be considered a work in progress.

10 - 	Change to have further control of conduit output in EXTRAN.  Version 4.4A change number 15 allowed flows in specified conduits to be written at specified time steps when flows are greater than a specified amount.  This was expanded to allow results for conduits to be written only when flows in reference conduits have flows greater than the specified minimum flow rate.  An application of this would be to write inflows to regulator and overflows from CSO regulator only when overflows are occurring.

11 -	Sediment depth option.  User can specify sediment depth for conduit and program adjusts hydraulic radius, area, and invert (ZP) appropriately.  Eliminates need to use equivalent pipes.  Implemented and tested only for circular conduits but should be easily expandable to other conduit types.  Program adjusts pipe invert elevations, normalized area curve, normalized top width curve and normalized hydraulic radius curve to account for sediment depth input.

12 - 	Do not open JIN in EXTRAN.FOR if not assigned to a file through @ line or command line input.

13 -	Do not open JOUT in EXTRAN.FOR if not assigned to a file through @ line or command line input.  This can save a lot of run time by avoid writing results to a scratch file that is not saved.  This avoids filling up hard disk during long simulations.

14 -	Modified EXTRAN full channel warning statements.  Old version wrote message 100 times then quit.  New program writes number of times full channel was exceeded in summary output.  Note that is a serious error.  Full channel area and hydraulic radius are used when depth of flow exceeds channel banks.

15 - 	Changes made to several subroutines to explicitly SAVE value of variables between subroutine calls.  Changes identified by CWRMB and CWRME in subroutines GUTTER, LINK, MELT, WRTFLOW

16 -	Cleaned up dimensioning in TRANSPORT for number of hydrographs.  Replaced NTH with NTHI, NTHO, NTHOA, NTHR.  NTHI is number of input hydrographs.  NTHO is number of output hydrographs.  NTOA is number of printed hydrographs and heads.  NTHR is number of input hydrographs on R lines.

17 - 	Changes made to implement parallel version using KAI parallel OMP directives.  These changes were made by William Magro of KAI, Inc.  These changes should not affect model results.

- Critical sections identified with C$OMP CRITICAL and C$OMP END CRITICAL.

- Changes to prevent parallel strings from trying to access same memory location.  NHEAD includes AS1 array to allow each instance to access separate memory locations when assigning conduit surface area to junctions.

- Various changes in XROUTE and YROUTE routines to uncouple parameters to avoid memory use conflicts between strings and to identify portions of the code that can be performed in series and those that can proceed in parallel.

18 - 	Write intermediate output file for MIKEVIEW.  The results are written to an internally documented, generic ASCII output file that could be used to further process EXTRAN output for other post processors or to analyze model results.

19 - 	Change to include entrance, exit, and additional losses in friction term when solving for flows in _ROUTE routines in EXTRAN.  Code provided by Bob Dickenson.  Note that this code is not correct when used with ISOL of 0 or 3 may produce erroneous results.  Strong message is provided and user has option to continue if desired.  Version 4.40 and 3.31 provided options to incorporate minor losses into equivalent Mannings n assuming full-flow. This takes this one step further.

20 -	Maximum conduit flows are input on C1 lines in EXTRAN for all types except 6, 7, 8, and 12.  IF SPHI and STHETA are zero, no change occurs.  If not zero, then SPHI is the maximum positive flow in the conduit.  If not zero, then STHETA is the maximum negative flow in the conduit.  Note that the user needs to be careful to ensure that values are not accidentally entered for these values or strange behavior may result.

21 -	Allow user to input x and y location of junctions on D1 lines in EXTRAN.  These are not used by EXTRAN but are passed to MIKESWMM intermediate interface file.  These values are available for other graphical pre or post processors to use to display model results.

22 -	Moved check for junctions connected to model to after all weirs, pumps, orifices have been read in EXTRAN.  Add check and make it an error if a junction is not connected to a conduit, pump station, weir, or orifice.

23 -	Prevent flat conduits in EXTRAN.  These are unstable for some reason.  If input as flat, add 0.001 feet to upstream Zp and provide warning message.

24 - 	Added feature to provide better description in output of what is going on in steady-state EXTRAN calculations.

25 -	Added VERSION subroutine to make it easier to update the version identification.  This includes release date and program used to compile the program.

26 -	Added SWMMDATE subroutine that contains all calls to system date and time.  These calls are typically compiler-dependent.  Subroutine includes Lahey and Digital Visual Fortran calls.

27 -	Corrected simulation duration calculation.

28 - 	Changes to EXTRAN HEAD routine.  Now called NHEAD in version 4.4E.  Part of the reason for the new file is described in change number 17.  Also made changes to improve stability when going into and coming out of surcharge.  Biggest change was to limit YNL and YNH to be less than or equal to crown of pipe and greater than or equal to zero.  Makes YMID calculation more accurate.  Includes changes require to implement M2 and S2 drawdown calculations as described in change #9.  Note, the changes to HEAD are described in greater detail later in this memorandum.

29 -	Changes to eliminate “+” from continuation location (column 6) in subroutines that start with P through Z.  This was causing problems in parallel version compiler until a work around was found.

30 -	Correction in RADH where M1 was used before value was set.

31 -	Print total rainfall and rainfall data starting and ending dates for each gage in RAIN block.

32 -	Change to RDII calculations in RUNOFF.  Program has been modified to allow input of R, T, and K parameters by month.

33 - 	In RUNOFF, add optional parameter IVCHAN to eliminate evaporation from channels.

34 -	Changes in TRANS to segregate PFILE into PFILEI read from interface file and PFILEO with data to write to interface file.  This array contains concentration data.  This change was made to allow these two arrays to be correctly dimensioned.

35 -	Changes in _ROUTE routines in EXTRAN to eliminate large continuity errors that could occur if the user specifies a negative constant inflow.

36 -	Change to STRIP routine to consider all input to the right of asterisks to be comments.  This is required on lines that have optional inputs.  The user should get in the habit of using asterisks to separate comments from data on an input line.

37 -	Limited testing was performed for the use of NEQUAL 4 or 5 (incorporate local losses in solution) with ISOL of 0 or 3 (standard explicit solution).  Program seemed to work correctly for the simple test cases that I used.  Based upon this test, I removed the check and warning against combining these two options.

38 - 	Expand JELEV option to input all data as depths instead of elevations.  0 read depths everywhere as in original program, 1 read elevations in place of depths for conduit Zp’s only.  JELEV equal 2 reads ZP reads conduit inverts and initial water surface elevations on D1 lines (Y0) as elevation in place of depth.  JELEV equal 3 reads conduit Zps, initial junction elevations on D1 lines, and surface area/elevation in place of surface area/depth on E2 lines.  JELEV equal 4 reads elevations in place of depths everywhere including:

Conduit invert elevations

Initial depth Y0 on D1 lines

Surface area/elevation rather than surface are depth on E2 lines

Orifice ZP is an elevation in place of depth

On F3 and F4, OOPEN and OCLOSE are elevations in place of depths

For weirs, YCREST and YTOP are elevations in place of depths

For pump types 2 and 4, VRATES are elevations in place of depths

For pump type 3, PON and POFF are elevations in place of depths

This option converts elevations to depths immediately.  No changes were made to model output.  This change may affect how some graphical interface programs interpret the EXTRAN data input file.  JELEV of 1, 2, or 3 will likely have little affect on the way that pipes are displayed.  Using JELEV of 4 will likely cause data to be displayed incorrectly by post processor programs.

39 -	As part of change number 38 above, inputs for variable orifices were modified slightly.  In previous version, the orifice invert could be made to change (as opposed to the default of the orifice crown) by entering the orifice ZP as a negative number.  This option allows  structures such as inflatable dams to be simulated.  Using negative ZP to indicate that invert should move as orifice opens and closes prevents the possibility of a negative invert elevation if JELEV equals 2 option is used (see change 38).  This was switched to have a negative orifice coefficient CORIF flag that the orifice invert will change.

40 -	When using irregular channel input for EXTRAN, the channel length appears at two places in the input file:  1) on the C1 lines, and 2) as part of the irregular cross section input line.  The program was modified to default to require that these two lengths be equal.  If they are not equal, an error message is generated and the program stops.  This avoids confusion and possible miss interpretation of the data input file.  Before this change, the program used the length read from the irregular channel data.  A variable IWLEN has been added to allow the behavior of the program to be modified.

IWLEN = 0 or not entered requires that the length be equal at both input locations.

IWLEN = 1 uses length on irregular section data as in the previous version of the program.  A warning message is generated if they are not equal.

IWLEN =2 uses length on the C1 line.  A warning message is generated if the lengths are not equal.

Note that GETCUR is also used by TRANSPORT irregular sections.  In TRANSPORT the two input lengths must be equal (e.g., IWLEN=0 option for EXTRAN).

41 -	Change made to allow STATS block to analyze negative flow events.  Useful if flows are negative when pipe slopes are adverse or if a pipe experiences both positive (storm flow) and negative (tidal or river inflow) events.  Added optional new parameter JNEG to A1 line.  If zero or not entered, then program operates as before.  If JNEG equals 1 then program process flow events whenever flows are less than BASE.  Events are excluded if the absolute value of the total event volume is less than EBASE.

- Changes to make number of constituents simulated consistent between all blocks (RUNOFF, COMBINE, TRANSPORT, STORAGE TREATMENT, and STATS.  Number of constituents in all blocks is set by MQUAL in TAPES.INC.  Eliminated NRQ that defined maximum number in RUNOFF.  Various changes were made throughout the program to increase dimensions and modify write statements.  Only change in input is the way that the program reads quality of constant manhole (Type 19) inflows to TRANSPORT Block.  Previously, constant inflow rate was input as DIST, and concentrations of four parameters were read as GEOM1, SLOPE, ROUGH, and GEOM2.  This was modified to have DIST still be the constant inflow rate.  If DIST is not equal to zero, then constant inflow concentrations are inserted on a new line that immediately follows the E1 line for the type 19 manhole element.  This line does not have a two character alphanumeric identifier, just NPOLL concentration values.  This line is required only for all E1 lines for type 19 manhole elements that have a non zero constant inflow rate.

- Make changes in all programs to print all 10 characters and digits in ID’s.

- Make changes in TRANSPORT to provide some degree of error checking for the connectivity of the modeled network.  The previous version did no error checking at all.  The user had to ensure that the network was set up correctly which is time consuming and could still result in major program errors.  Program checks that upstream elements are in the model, that there are no loops in the system, that a element is referred to as an upstream element only once (or twice if it is a flow split type element).

- Introduce new type of element to TRANSPORT.  Type 27 is referred to as a quality splitting element.  This non-conduit type element allows the user to enter the “delivery ratio” for each quality constituent being simulated.  This is used to simulate the effect of water quality BMP’s in the system.  Only constituent mass is removed, not flow.  This element should be placed immediately downstream from flow routing storage unit that represents the element (if flow routing storage is simulated).  A flow split element can be placed immediately upstream from the water quality splitting element to simulate possible increase in delivery ratio during high flow events.

- Modify Storage/Treatment block to allow more that five units to be simulated.

- Provide on-screen summaries of continuity errors for all blocks at end of simulation.

- Rain Block - Increase number of storms to 20,000.

- Implement bolted manhole rim option in EXTRAN.  Option allows a second manhole rim elevation to be entered.  Model will allow HGL to rise to this second elevation before overflows will occur.

- Implement link between EXTRAN and WASP.  As part of this change, I experimented with alternative continuity check that is printed out after the regular check.  This new check may be more accurate in case with irregular channels or variable surface area storage nodes.

- Add ejector type pump station that turns on when wet well is full and turns off when wet well empties.

52	Program gives an error message and stops computations if stations on NC lines for changes in Mannings N do not correspond exactly to channel cross section stations.

COMPARISON OF MODEL RESULTS

Most of the program changes incorporated into version 4.4G deal with the EXTRAN block. EXTRAN simulations were made for a few select simulation to compare the simulation results for a few select cases. In additional to the two Versions compared in this memorandum, results were also compared with SWMM Version 4.31 EXTRAN.  Due to the complexities of the model, and of various input data files, it is not possible to provide detailed descriptions of differences in the results produced by the three SWMM Versions.  Also, the findings for these simple test cases may not carry over into complex systems.  The general findings of comparisons of model results are discussed below.

Model results were found to be identical for simple models that had open channel flow for steady state conditions.  Differences begin to appear for closed conduit systems that go into surcharge.  The differences appear to be related to closed conduits that are surcharged at the upstream end and are not surcharged at the downstream end.  These differences can be traced to changes made to subroutine HEAD.  The calculations performed in subroutine HEAD are discussed further below.

In general, the results produced by SWMM Version 4.4G tend to be more stable than the previous versions when going into and coming out of surcharge.  The model also does not seem to have instabilities when going from a dry pipe condition.

In most cases, the maximum junction water surface elevations produced by the three models will be within a few tenths of a foot of one another.  In some cases, the differences may be greater.  In many cases, the reported peak in one or the other model will result from model instabilities.  My experience with the models suggests that the revised program is more stable for most cases.

## CALCULATIONS PERFORMED IN SUBROUTINE HEAD

Subroutine head performs two major functions:

1 	Assign conduit surface area to upstream and downstream junctions.

- Compute “average” conduit depth, hydraulic radius, width, and cross-sectional areas used to solve for conduit flow.

The changes made to head should not affect the calculations related to the assignment of surface area to the junctions.

The following presents a somewhat simplified description of the old and new way that average conduit hydraulic parameters are computed in the HEAD routine (NHEAD in Version 4.4G).

Head is called to compute parameters for a single conduit.  The two main parameters passed to head are the water surface elevation at the upstream and downstream junctions.  The depth at the upstream (YNL) and downstream (YNH) end of the conduit are computed by subtracting the pipe invert elevation Zp.

In version 4.31,  HYDRAD was called to computed area, radius, and top width for depths corresponding to YNL and YNH:

Upstream:  	YNL, ANL, RNL, and BNL

Downstream:	YNH, ANH, RNH, and BNH

If depth of flow is greater than the conduit depth, then area, hydraulic radius and the top width equal the full-channel values.  If Y is less than zero then area, radius, and width are returned as zero from HYDRAD.

“Average” values used to represent flows in the conduit are computed as the arithmetic average:

AREA = 0.5 * (ANL + ANH)

HRAD = 0.5 * (RNL + RNH)

WIDTH = 0.5 * (BNL + BNH)

YMID = 0.5 * (YNL + YNH)

Version 4.4G takes a different approach.  In this revised version, YNL and YNH are limited to 1) being greater than or equal to zero, and 2) less than or equal to the conduit depth.  YMID is calculated as the arithmetic average of the upstream and downstream depths.  Then, HYDRAD is called to compute the area, hydraulic radius, and top width that correspond to YMID.  In this case, the area, radius, and width are all consistent with the average depth.  In the older version of the model, average area, radius, and width may not be consistent with the average depth.

The differences between the two techniques are subtle.  It is difficult to say which is more correct.  In a rectangular channel, the results are the same.  However, for other shapes the average of the areas is likely not equal to the area corresponding to the average depth.  In the few cases that I have simulated, it appears that the revised model produces slightly lower heads

This averaging of element hydraulic properties is the basis for the need for the IM2 correction.  Using midpoint values to represent channel characteristics is valid in many cases.  However, in certain gradually varied water surface profiles, the midpoint value appears to underestimate the conveyance capacity of the channel.  This results in the “damming” effect that can be observed in some cases.  Brett Cunningham’s solution attempts to define a table of weighting factors other than 0.5 to use to compute the average depth (YMID) for certain drawdown conditions.