## Memorandum

To:	Wayne Huber

From:	Chuck Moore

Date:	September 8, 2000

Subject:	Changes to SWMM Programs

On  September 8, 2000 I did a line by line comparison of the present CDM version of the code and the version of the code downloaded from your HTP site on September 8.  I incorporated your code changes into the CDM version of the code or vice versa.  The code includes all of your changes plus the CDM additions.

The following documents changes in the CDM version of the program that do not exist in your code.  The four major program modifications are:

- TRANSPORT IRREGULAR SECTIONS :   By default, TRANSPORT subdivides sections into 25 equal-area segments. For irregular sections that include large overbank areas, the lowermost equal area-segment can include the entire incised stream channel.  An option was added to break the lowermost equal-area segment into 25 additional segments.  This option is invoked by IDETAIL on B0 input line.  This is documented in TRANS.DOC.  Most useful if you are driving WASP from TRANSPORT or are using TRANSPORT velocities to evaluate stream velocities and scour potential.

- TRANSPORT DEPTH AND VELOCITY:  Add option to have TRANSPORT output head and velocity to the interface file for statistical analysis.  Controlled by negative NPOLL on B1 line.   Documented in TRANS.DOC.

- EXTRAN CRITICAL ELEVATION COMPARISON:  Add optional D2 and D3 line to read elevations against which junction elevations will be compared and reported.  Documented in EXTRAN.DOC

- RDII METRICATION:  Changes to make RDII calculations and continuity check correct when working in metric units.  I notice that Dr. Huber did some changes also.  In this version area remains in hectares and rainfall in mm.  Flows are computed in CFS.  I’m pretty sure that my changes work.  Continuity error check is correct also.

The following table summarizes the changed subroutines and the reasons for the changes:

| File Name | Reason for Changes |
| --- | --- |
| CHECKSTCH.FOR | New subroutine to check STCHL and STCHR. Called from GETCUR. |
| DEPTH.FOR | Change #1. |
| ERROR.FOR | Various changes to try to make error messages consistent. |
| EXTRAN.FOR | Add “warning” to error message to allow text search to find. |
| EXTRAN.DOC | Documentation for change #3. |
| FINDA.FOR | Change #1. |
| GETCUR.FOR | Change #1. Note major error in power function where hydraulic radius is computed before area is. Guess no one uses this option. |
| GETHV | Computes depth and velocity for output to interface from TRANSPORT, change #2. |
| GRAPH.FOR | Changed NQP to NQPP to avoid confusion. |
| HUGO.INC | Add NPOLL2 for change #2. Move parameter statement to TAPES.INC |
| HYDRO.FOR | Metric units conversion for RDII calculations, change #4 |
| INDAT1.FOR | Move opening of interface file until we know start and end dates in subroutine INDAT3. Change #3. |
| INDAT3.FOR | Add error checking for K_ lines. Open interface file here. |
| INFACE.FOR | Check interface locations against array dimensions. |
| INTRAN.FOR | Change#1 and Change#2. |
| File Name | Reason for Changes |
| IRRSECT.FOR | New subroutine that does irregular section calculations previously performed in GETCUR. Avoids duplication of code. Change #1 |
| NEWTR.INC | Add IDETAIL to common block. Change #1. |
| OUTPUT.FOR | Output threshold table (Change # 3). Output additional hours. |
| RDIIREAD.FOR | Change to correct metric version of RDII calculations, change #4 |
| RDIIRES.FOR | CIM’s correction for metric version, change #4. Also changed JSTA to character*8 to match other programs. Change comparison of time steps to not be absolute. |
| RYDRO1.FOR | Added some additional checks and cleaned up one format. |
| ROUTE.FOR | Changes for additional detail for TRANSPORT irregular sections, change #1. |
| SBTABL.FOR | Change to print only one header per table, not multiple. Slight fix to table format 265 and 266. |
| STATS.FOR | Correction to look at KTIME instead of KREAD. |
| STRIP.FOR | Change to allow really long input lines. |
| TAPES.INC | Add MTHRESH for change 3. Note that dimensions are significantly different in this version. Set parameter NQP equal to NET to make code cleaner. Remove from HUGO.INC. |
| THRESHOLD.FOR | Contains subroutines added for change number 3. |
| THRESHOLD.INC | Common block for change number 3. |
| TIDEFILE.FOR | Add checks to make sure that boundary conditions at junctions are defined properly. |
| TRANNORM.FOR | Subroutine added to compute normalized statistics for TRANSPORT elements. Called by GETCUR. |
| TRANS.FOR | Changes to output head and velocity to interface file. |
| TRANSX.FOR | Call to COMPTHRESH to compare simulated elevations against critical elevations, change #3. |
| WASP.FOR | Add additional warning messages and added correction for metric SWMM simulations. Note, WASP is metric. |